Figure 1: The HE CellProfiler pipeline workflow. A. The original image (left) is masked and normalized using slideEMask and slideNormalize in the slideToolKit workflow and used as input by CellProfiler; the graph (right) shows the tonal distribution in the digital whole-slide image on a RGB scale. B. The input image is converted to a gray scaled image (left); the graph (right) shows the tonal distribution in the gray scaled image. C. A Gaussian filter is applied to smoothen the image and reduce image artefacts (artifact size 20 pixels) and noise (left); the graph (right) shows the tonal distribution after smoothening. D. The gray scaled image is inverted, i.e. non-tissue will become black (left); the graph (right) shows the tonal distribution after inverting. E. The tissue area is identified, as demarcated by the green line in the left image; the total tissue area size is calculated in pixels (right image) and tabulated (table).  F. The colors, i.e. stains, are unmixed using the original image (left): Hematoxylin (HE, middle), and Eosin (right). G. The HE-positive objects, i.e. nuclei, are identified, white areas in the left image; the HE-positive objects are demarcated by a green line in the middle image, areas that are excluded due to size (minimal size 12 pixels) are demarcated in magenta; the right image shows all the identified HE-positive objects in random colors; the total number of identified objects is calculated and tabulated (table). H. Finally the data for each tile are saved in a comma-separated table, including meta-data such as tile positions, image location, object counts (there could be multiple patches of stained areas or tissue). The original image (left) is used to overlay the HE-positive objects (demarcated with blue) and the tissue area (demarcated with green). Entropy threshold used for slideEMask: 210. Sample used: AE1468.UMC.HE.ndpi.